Networking

Assignment

Report

Submitted By

Name: Serena Yonjan

Uno.id : 20251095

Submission date:10/30/2025

Question 1:

Scenario

Suppose a small Research Center of ABC university has two Computer Labs. First lab,

PC_LAB_1 which consists of three computers and one server connected with each other

through a switch which is further connected to a router. Similarly , Second lab, PC_LAB_2

which consists of four computers and one server connected with each other through a

switch which is further connected to a router. Both the routers of PC_LAB_1 and PC_LAB_2,

are further connected with each other to create a network based on routing information

protocol RIPv2.

Considering yourself as the Network Administrator of the University Research Center, you

are required to perform following tasks using the Packet Tracer Tool.

Take screenshot after completing each activity /task and paste in the MS Word File after

completing the tasks:

Task1:

Task 2:

LAB1_Router Configuration

PC1_LAB1 Configure:

LAB1_Server Configure:

LAB2_Router Configuration:

PC1_LAB2 Configure:

LAB2_Server Configure:

Task 3:

RIPv2 Configuration – LAB1_Router:

RIPv2 Configuration – LAB2_Router:

Task 4:

Passive Interface – LAB1_Router:

Passive Interface – LAB2_Router:

Task 5:

LAB1_Router:

Show Ip route and Ip protocols:

LAB2_Router:

Show Ip route and protocols:

Task 6:

Question 2:

Introduction :

Router can learn of remote networks via static or dynamic routing. This activity focuses on how

remote networks are added to the routing table using static routes. This is one of two ways that

a remote network can be added to the routing table. Static routes are configured by the

networks administrator and include the network address and subnet mask of the remote

network, along with the IP address of the Next-hop router or the exit interface of the local

router.

Configure static Routing

| Device | Interface | Ip address | Subnet mask | Gateway |
| --- | --- | --- | --- | --- |
| R1 | F0/0 | 192.168.100.1 | 255.255.255.0 | - |
|  | S0/0/0 | 192.168.110.1 | 255.255.255.0 | - |
| R2 | S0/0/0 | 192.168.100.2 | 255.255.255.0 | - |
|  | S0/0/1 | 192.168.200.1 | 255.255.255.0 | - |
|  | F0/0 | 192.168.120.1 | 255.255.255.0 | - |
| R3 | S0/0/0 | 192.168.200.2 | 255.255.255.0 | - |
|  | F0/0 | 192.168.130.1 | 255.255.255.0 | - |
| PC1 | NIC | 192.168.110.2 | 255.255.255.0 | 192.168.110.1 |
| PC2 | NIC | 192.168.120.2 | 255.255.255.0 | 192.168.120.1 |
| PC3 | NIC | 192.168.130.2 | 255.255.255.0 | 192.168.130.1 |

Task 1: Configure static route on R1

Task 2: Configure static route on R2

Task 3: Configure static route on R3

Task 4 : Check the connectivity

Ping from PC1

Ping from PC2

Ping from PC3

Question 3:

Configuring VLANs and Trunks.

Topology Diagram

Task 1: View the Default VLAN Configuration

a. Verify the current running configuration on the switches.

Switch 2:

Switch 0:

Switch 1:

b. Display the current VLANs.

Switch 2:

Switch 0:

Switch 3:

c. Verify connectivity between PCs on the same network.

PC0-PC3

PC1-PC4

PC2-PC5

Task 2: Configure VLANs

a. Create VLANs on S0

b. Verify the VLAN Configuration

Create VLANs on S2

Verify the VLAN Configuration

Create VLANs on S1

Verify the VLAN Configuration

Task 3: Assign VLANs to Ports

a. Assign VLANs to the active ports on S0

b. Assign VLANs to the active ports on S1

c. Verify loss of connectivity

So we notice, PC0 is unable to ping PC3. PC0 and PC3 may not be able to ping each other when ports are moved to particular VLANs. Reason: Because there is no Layer-3 routing across VLANs by default and access ports place hosts into different VLAN broadcast domains, frames for those VLANs may not transit between switches if linking between switches isn’t configured.

Task 4: Configure Trunking

Configure S1 Fa0/1 and Fa0/3 for trunking and to use VLAN 99 as the native

VLAN

b. Verify connectivity between devices on the same VLAN.

Although there is currently a native VLAN mismatch, pings between PCs on the same VLAN are

now successful. Why?

Devices on VLAN ping are visible. Pings between PCs on the same VLAN are successful despite the current native VLAN mismatch because VLAN data traffic is tagged. Only in tagged frames are impacted by the mismatch. Successful communication is made possible by accurately identifying and forwarding tagged VLAN traffic, such as that from PCs in VLAN 10, over the trunk link.

d. Verify trunking is enabled on S1 and configure VLAN 99 as the native VLAN.

Verify trunking is enabled on S2 and configure VLAN 99 as the native VLAN.

Question 4:

SSH should replace Telnet for management connections. Telnet uses insecure plain text

communication. SSH provides security for remote connections by providing strong encryption of

all transmitted data between devices. In this activity. You will secure a remote switch with

password encryption and SSH.

Instructions

Task 1: Secure passwords

Using command prompt on PC1. Telnet to S1. The user EXEC and privileged EXEC

password is Naami

• Save the current configuration so that any mistakes you might make can be reversed by

toggling the power for S1.

• Show the current configuration and note that the passwords are in plain text. Enter the

command that encrypts plain text passwords.

• Verify that the passwords are encrypted.

Task 2: Encrypt Communications

a. Set the IP domain name and generate secure keys.

• It is generally not safe to use Telnet. Because data is transferred in plain text.

Therefore, use SSH whenever it is available.

• Configure the domain name to be padmashree.com

• Secure keys are needed to encrypt the data. Generate the RSA keys using a 1024

length.

b. Create an SSH user and reconfigure the VTY lines for SSH only access.

• Create an administrator user with cisco as the secret password.

• Configure the VTY lines to check the local username database for login credentials

and to only allow SSH for remote access. Remove the existing vty line password.

Task 3: Verify SSH Implementation

• Exit the telnet session and attempt to log back in using telnet. The attempt should fail.

• Attempt to log in using SSH command.